MPHM1144

- The MPH2TM Type M measures the electrical parameters per phases and individual outputs.

- Adjustable threshold values with an alarm function increase the availability of the connected loads.

- The integrated RPC2k communications card allows you to connect together up to 4 MPH2TM / MPXTM units and to remote-monitor and maintain all connected units. This card can be replaced during operation.

- The MPH2TM unit has an integrated display. In addition, the optional external RPCBDM-1000 display module can be connected and flexibly attached to the rack.

- Up to 10 sensors (temperature, air humidity, door contacts as well as potential-free input contacts) can be connected to the communications card.

- Cables with locking mechanisms can be connected to the outputs.

- The PDU can be integrated into ENP Knürr racks or other housing systems with corresponding slots without the use of tools.

Remote management: Onboard web interface, CLI, SNMPv1,2,3, SSH, Telnet, integratable into Avocent ACS, UMG & MPU, DSView, Rack Power Manager, Nform & Trellis as well as Nagios or other management software programs

Authentication: Local, remote: Active directory, LDAP, TACACS, Radius, Kerberos

Encryption: MD5, AES, DES

Measurement parameters: Current (A), voltage (V), real power (W), apparent power (VA), consumption (kWh), power factor, frequency and crest factor

Metering accuracy: +-1% (V, A); Power loss: 3W - 5W; Operating temperature: +60°C

Load rating

Input: plug IEC60309 3ph/N/PE 6h, 230/400Vac / 16A

Output: socket-outlets

18 x IEC60320 C13, max. 10A per outlet

6 x IEC60320 C19, max. 16A per outlet

Dimensions

- Length: 1736.5 mm

- Width: 56 mm

- Height: 50 mm

- Cable: 3 m

Approvals

- CE-mark

- CB certification

- BG certification

Material / surface / color

- Housing: closed aluminum profile, finely textured powder-coated top (RAL 7021)

- Plastic parts: Vampamid 6 0024 V0 (UL94), recyclable, finely textured and colored (RAL 9005)

Supply schedule

1 MPH2TM socket strip (PDU)

2 Mounting brackets standard

2 Toollessmounting buttons

1 Quickstart Guide

How supplied

- flat-packed kit